[image: image1.png]VERBAL INSTRUCTIONS TO REPLACE A 1600119 DC-TO-DC CONVERTER
WITH A SEVCON 1600122 DC-TO-DC CONVERTER.

REMOVE THE BLACK 10 GAGE NUMBER 1, AND THE BLACK 10 GAGE NUMBER 2 WIRES FROM TERMINAL STRIP NUMBER 2 B-
OF THE 1600119 CONVERTER.
(THESE WILL BE REINSTALLED IN THE LAST SEP)

REMOVE THE RED 10 GAGE NUMBER 2 WIRE FROM TERMINAL NUMBER 1 B+, AND THE RED 10 GAGE JUMPER WIRE FROM
TERMINAL 1 B+ TO 3, DISCARD THIS RED 10 GAGE JUMPER WIRE.

INSTALL THE RED 10 GAGE NUMBER 2 WIRE TO THE TERMINAL MARKED + 36/48 (FAR LEFT TERMINAL) OF THE 1600122
CONVERTER.

REMOVE THE RED 10 GAGE NUMBER 3 WIRE FROM TERMINAL NUMBER 1 +12, AND THE RED 10 GAGE JUMPER WIRE FROM
TERMINAL 1 +12 TO 2 +12, DISCARD THIS RED 10 GAGE JUMPER WIRE.

INSTALL THE RED 10 GAGE NUMBER 3 WIRE TO THE TERMINAL MARKED + 12 (3% FROM LEFT) OF THE 1600122
CONVERTER.

INSTALL THE TWO WIRES THAT WERE REMOVED IN THE 1" STEP TO THE TERMINAL MARKED - 36/48 (2ND FROM LEFT) ON
THE 1600122 CONVERTER.

ADD A 10 GA BLACK WIRE FROM THE TERMINAL MARKED - 12 (FAR RIGHT) TO THE SAME GROUND POINT AS THE BLACK 10

GAGE NUMBER 2 WIRE MENTIONED ABOVE.
NOTE: ON A UL MACHINE THIS WILL NOT BE LOCATED ON THE CHASSIS, BUT WILL BE TO THE NEGATIVE TERMINAL OF

THE BATTERY.




